Biased Edging and Mitred Corners

- Put right sides of quilt and biased edging together, pin and sew biased edging on in the first crease, leaving approximately 2cm unstitched at each end at this stage.

First crease

only sew to where I have marked these arrows to begin with.

Biased edging at a 90 degree angle

For the longer edges, you will need to join an extra piece of biased edging to the end to make it long enough to fit onto the longer sides of the quilt. This must be done with a biased join. To do this; place right sides together of two lengths of biased edging at a 90 degree angle to each other, leaving 0.5cm of over hang. Stitch a straight line from corner to corner to then straighten it out to be one longer strip of biased edging, trim off triangle shaped excess fabric from seam and iron flat.

This is the sewing line for the biased join, trim off excess fabric on edge.

- Once all the edging is sewn on, you need to finish the stitching on the biased edging so they meet up right at the same point, this will only be a matter of sewing an extra centimetre or so

This triangle represents where the stitching will need to meet and be backstitched to hold it in place. Right in the middle of this line made by the trianglewhich the arrow is sitting on.

This point is where the seams have beem matched up

Once you have made all corners meet up, you need to do your corners. You need to pull your biased edging pieces out so they are right sides together.

- Once this has been done, mark the point at which the stitching meets. Which is at the first crease. Stitch from there at a 45 degree angle to the sewing line on the biased edging to the second crease. Now stitch from this point at a 90 degree angle from the line just stitched to the third crease, backstitch.

This arrow sits right on the spot where the stitching on the first crease meets up and points to the stitching line that need to be sewn. Now trim off the excess fabric. Now turn the biased edging over to the back side of the quilt and slip stitch this to the back. Remember to only pick up 1 thread at a time to try to minimise what you see of the hand stitching, also don’t double up the cotton and tie a knot in the end, use a single strand of cotton.